KULLIYYAH OF INFORMATION AND COMMUNICATION TECHNOLOGY

DEPARTMENT OF INFORMATION SYSTEMS

INFO 4401 REPORT

ELERA: AN INTELLIGENT WEB-BASED ARDUINO CIRCUIT DESIGN ASSISTANT

MOHAMAD IMAD ADDIN BIN JA'FAR

2127923

SUPERVISED BY

DR AHMAD FATZILAH BIN MISMAN

Report submitted in fulfilment of the requirements for Bachelor of Information Technology (Hons)

Kulliyyah of Information and Communication Technology

SEPTEMBER 2025

SEMESTER 2, 2025/2026

# ABSTRACT

ABSTRACT

Arduino has become the go-to platform for learning electronics in universities, but beginners still struggle with common wiring mistakes like forgetting ground connections, pulling too much current, or plugging things into the wrong pins. Tools like Tinkercad, Wokwi, and Fritzing let students build circuits virtually, but they don't actually check for these mistakes while the student is designing. The errors only show up later, during simulation or physical testing, and even then there's no explanation of what went wrong. Elera was developed to fill that gap. It is a web-based Arduino circuit design assistant built with Lit 3, Vite 6, and the @wokwi/elements library. The system gives students a drag-and-drop canvas to build circuits, a metadata-driven component library covering 14 common Arduino parts, a validation engine that checks for nine categories of electrical errors in real time, and an auto-wiring engine that can connect components to the Arduino board automatically with the right pin assignments. Development followed an Agile Iterative approach across seven sprints. Elera runs entirely in the browser with no installation required. What makes it different from existing tools is that it doesn't just flag errors, it explains the problem and tells the student how to fix it, turning the circuit builder into something closer to a learning tool.

# TABLE OF CONTENTS

TABLE OF CONTENTS

CONTENTS								                PAGE

SUPERVISOR APPROVAL	ii

CERTIFICATE OF ORIGINALITY	iii

ACKNOWLEDGEMENT	1

ABSTRACT	2

TABLE OF CONTENTS	3

LIST OF FIGURES	7

LIST OF TABLES	8

1.0	PREAMBLE	9

1.1	PROBLEM DESCRIPTION	9

1.1.1	Background of the problem	9

1.1.2	Problem Statement	9

1.2	PROJECT OBJECTIVE	10

1.3	PROJECT SCOPE	10

1.3.1	Scope	10

1.3.2	Target Audience	10

1.3.3	Specific Platform	10

1.4	CONSTRAINTS	10

1.5	PROJECT STAGES	10

1.6	SIGNIFICANCE OF THE PROJECT	11

1.7	SUMMARY	11

CHAPTER THREE: METHODOLOGY	12

2.0	INTRODUCTION	12

2.1	SYSTEM REVIEW DIGITAL STORYTELLING/E-LEARNING APPLICATION/GAME REVIEW (Please remove the unnecessary).	12

2.1.1	Existing System/ Digital Storytelling/E-Learning Application/Game Development (Please slash the unnecessary).	12

2.1.2	Advantages	12

2.1.3	Disadvantages	12

2.2	DISCUSSION	13

CHAPTER FOUR: RESULTS AND FINDINGS	14

3.0	INTRODUCTION	14

3.1	DEVELOPMENT APPROACH	14

3.2	REQUIREMENTS SPECIFICATION	14

3.3	LOGICAL DESIGN	14

3.3.1	System Analysis and Design Diagram	14

3.3.2	Data Flow Diagram (if applicable)	14

3.4	DATABASE DESIGN	15

3.5	USER INTERFACE/PROTTYPE DESIGN	15

3.6	SUMMARY	15

CHAPTER FOUR: RESULTS AND FINDINGS	16

4.1	INTRODUCTION	16

4.2	REQUIREMENT SPECIFICATIONS/DATA COLLECTIONS	16

4.2.1	The Findings	16

4.3	CONCEPTUAL DESIGN	16

4.4	STORY STRUCTURE	16

4.4.1	Story Plot/Script	16

4.4.2	Character Design	17

4.5	STORYBOARD	17

CHAPTER FOUR: RESULTS AND FINDINGS	18

3.0	INTRODUCTION	18

3.1	DEVELOPMENT APPROACH	18

3.2	REQUIREMENT SPECIFICATION/DATA COLLECTIONS	18

3.2.1	The Findings	18

3.3	THE NEEDS ANALYSIS	18

3.4	THE APPLICATION DESIGN	18

3.5	CONTENT DEVELOPMENT	18

3.6	NAVIGATION FLOW OF THE APPLICATION	19

3.7	STORYBOARD	19

CHAPTER FOUR: RESULTS AND FINDINGS	20

3.0	INTRODUCTION	20

3.1	DEVELOPMENT APPROACH	20

3.2	STRUCTURE	20

3.2.1	Story Plot	20

3.2.2	Character Design	20

3.3	STORYBOARD	20

CHAPTER FOUR: RESULTS AND FINDINGS	21

4.0	INTRODUCTION	21

4.1	SYSTEM INTEGRATION	21

4.2	SYSTEM OUTPUT	21

4.2.1	Administrator	21

4.2.2	User(s)	21

4.3	SYSTEM TESTING	21

4.3.1	Test Plan	21

4.3.2	Enhancement	22

CHAPTER FOUR: RESULTS AND FINDINGS	23

5.1	INTRODUCTION	23

5.2	FINAL COMPOSITION (Production)	23

5.2.1	Digitizing Art	23

5.2.2	2D//3D Modelling and Animation	23

5.3	RECORDING AND DIGITIZING AUDIO	23

5.4	IMPLEMENTATION	24

5.5	EVALUATION/TESTING (Post-Production)	24

5.5.1	Test Plan	24

5.5.2	Enhancement	24

CHAPTER FOUR: RESULTS AND FINDINGS	25

6.1	INTRODUCTION	25

6.2	APPLICATION DEVELOPMENT (Production)	25

6.2.1	Digitizing 2D/3D Art	25

6.2.2	Digitizing Scenes	25

6.2.3	Recording And Digitizing Audio	25

6.2.4	Authoring	25

6.3	IMPLEMENTATION	25

6.4	EVALUATION/TESTING (Post-Production)	26

6.4.1	Test Plan	26

6.4.2	Enhancement	26

CHAPTER FOUR: RESULTS AND FINDINGS	27

7.0	PROJET REQUIREMENT	27

7.1	PROJECT CONSTRAINT	27

7.2	FUTURE ENHANCEMENT	27

7.3	CONCLUSION	27

REFERENCE	28

APPENDICES	29

# LIST OF FIGURES

LIST OF FIGURES

FIGURE									        PAGE

Figure 1 Scene 1.	23

# LIST OF TABLES

LIST OF TABLES

TABLE									PAGE

Table 2-1 A Table	13

# CHAPTER 1

# INTRODUCTION

## PREAMBLE

Arduino has become one of the most popular platforms for learning electronics and embedded systems. It's used in universities, schools, online courses, and by hobbyists all over the world. The appeal is clear: it's open-source, affordable, and there's a massive community around it. But for someone just starting out, actually wiring a circuit on a breadboard can be surprisingly difficult. Getting the code right is one thing, but understanding which pin goes where, why a resistor is needed, and how much current a component draws is a whole different challenge. A lot of beginners end up learning through trial and error, which is slow and sometimes results in burnt components. There are web-based tools that help with this, like Tinkercad and Wokwi, but most of them are simulators. They let users build and test circuits, but they don't catch wiring mistakes during the design process or explain what the student did wrong. That gap between just simulating and actually guiding the learner is what Elera was built to address.

## PROBLEM DESCRIPTION

Learning Arduino circuit design comes with a lot of friction, especially for people doing it for the first time. The problems aren't just about knowledge gaps; they're also about the tools available.

### Background of the Problem

Most beginners pick up Arduino wiring from YouTube tutorials, blog posts, and forum threads. They follow along step by step, and when it works, great. But when it doesn't, they're stuck trying to figure out why. Common mistakes include connecting an LED straight to 5V without a resistor (which burns it out), wiring two components to the same signal pin, exceeding the Arduino's 500 mA current limit, or plugging an I2C device into the wrong analogue pins. These errors can damage real hardware and produce confusing behaviour that's hard to debug, especially for someone who doesn't yet have a strong grasp of the underlying electrical principles.

Several popular tools exist to help. Tinkercad Circuits, built by Autodesk, offers a free browser-based simulator where users can build circuits and run Arduino code (Autodesk, 2017). Wokwi provides a similar web-based simulation environment with support for Arduino, ESP32, and other boards (Wokwi Ltd., 2020). Fritzing is a desktop application focused on breadboard layout and PCB design (Friends-of-Fritzing, 2009). These tools are genuinely useful, but they share one important limitation: they're passive. The user builds the circuit manually, and errors only become visible when the simulation runs, or worse, when the physical circuit fails. None of them actively scan for wiring problems during the design process. And none of them tell the user why something is wrong or how to fix it.

That's the core gap. There's a big difference between a tool that simulates a circuit and one that actually teaches someone how to wire it correctly.

### Problem Statement

Four specific problems were identified:

1.  Beginners regularly make electrical wiring errors (missing grounds, overcurrent, wrong pin types, floating pins), and existing tools don't catch these during the design phase. The mistakes only surface during simulation or physical testing, which makes finding the root cause much harder.

2.  When errors do appear, current tools don't explain them. There's no feedback saying "you're missing a resistor because the LED will draw too much current without one." Students are left to figure it out on their own.

3.  Every existing web-based Arduino circuit builder requires fully manual wiring. The user has to know, for example, that a servo needs a PWM pin, or that an I2C LCD goes on A4 and A5. For beginners who don't know these rules yet, it's a tedious and error-prone process.

4.  No single accessible web application brings together visual circuit building, real-time rule-based validation, and automated pin assignment in one place. Some of these features exist in isolation across different tools, but nobody has combined them.

## PROJECT OBJECTIVE

The main goal of the project is to design and develop Elera, a web-based Arduino circuit design assistant that gives beginners proactive guidance instead of just a blank canvas. The specific objectives are:

1.  To design and develop a web-based Arduino circuit builder with drag-and-drop component placement and manual pin-to-pin wiring on an interactive canvas.

2.  To implement a rule-based validation engine (the Elera Validation Engine, or EVE) that detects common electrical errors, including missing power/ground connections, current overload, duplicate pin assignments, wrong I2C wiring, floating pins, and missing current-limiting resistors, and gives immediate, beginner-friendly feedback.

3.  To develop an auto-wiring engine that assigns the correct Arduino pins to component pins based on their electrical requirements (digital, analogue, PWM, I2C) and generates the wire connections automatically.

4.  To create a metadata-driven component library where each component's pin roles, current draw, and auto-wiring rules are stored as structured data, so the system can be extended with new components without touching the core logic.

## PROJECT SCOPE

### Scope

Elera is a client-side web application built around the Arduino Uno. It includes:

•  A component library with 14 beginner-level parts: Arduino Uno, LED, RGB LED, resistor, push button, potentiometer, servo motor, buzzer, LCD 1602 (I2C), HC-SR04 ultrasonic sensor, DHT22 temperature sensor, PIR motion sensor, NeoPixel LED, IR receiver, and slide switch.

•  A visual canvas with drag-and-drop placement, pin-to-pin wiring, grid snapping, anti-overlap detection, wire waypoint editing, undo/redo, and pan/zoom.

•  A validation engine covering nine rule categories: missing board, LED without resistor, current overload, duplicate pin assignment, servo on serial pins, missing power/ground, wrong I2C pins, unconnected components, and floating pins.

•  An auto-wiring engine that assigns pins based on type and creates the connections.

•  Local saving via browser localStorage.

What's not included:

•  Full circuit simulation or running Arduino code.

•  PCB design or schematic capture.

•  Support for other boards (ESP32, Raspberry Pi Pico, etc.).

•  Backend, user accounts, or cloud storage.

•  AI-driven circuit generation (planned for later).

### Target Audience

The primary audience is undergraduate students learning electronics or embedded systems, particularly those who are still getting comfortable with circuit wiring. The secondary audience is hobbyists and self-learners who are just starting with Arduino.

### Specific Platform

Elera runs entirely in the browser and doesn't need a server for its core features.

Development environment:

•  Windows 10/11, Node.js 18+, Vite 6.0

•  Lit 3.2 (Web Components), @wokwi/elements 1.9.2

•  JavaScript (ES Modules)

User environment:

•  Any modern browser (Chrome, Firefox, Edge)

•  No installation or plugins needed

•  Minimum resolution: 1280 x 720

## CONSTRAINTS

1.  Only the Arduino Uno board is supported. Other boards like ESP32 or Arduino Mega aren't included in this version.

2.  Component visuals depend on the @wokwi/elements library. If a component isn't available there, it can't be shown in Elera without building custom rendering.

3.  The validation engine runs on predefined rules, not physics-based simulation. It catches structural wiring errors but can't simulate voltage levels, signal timing, or code-dependent behaviour.

4.  Everything runs in the browser. There's no backend, no database, and no login system, so projects only exist in the browser's local storage and can't be shared across devices.

5.  AI integration with Google Gemini is planned for future phases but isn't part of this deliverable.

## PROJECT STAGES

Development followed an iterative approach with these milestones (a Gantt chart is in the Appendices):

1.  Literature review and requirements gathering. Reviewing existing tools, spotting gaps, and defining what Elera needs to do.

2.  System architecture design. Planning the modular structure, state management, and how data flows between components.

3.  Core canvas development. Building the interactive SVG canvas with placement, grid snapping, pan/zoom, and anti-overlap.

4.  Component library and sidebar. Creating the metadata-driven catalog and the searchable sidebar UI.

5.  Manual wiring system. Implementing pin-to-pin wiring with orthogonal routing, waypoints, and colour-coded wires.

6.  Validation engine. Developing and integrating the nine-rule engine with the validation bar.

7.  Auto-wire engine. Building the pin assignment algorithm.

8.  Prototype completion and testing. Final integration, UI polish, and functional checks.

9.  Report writing.

## SIGNIFICANCE OF THE PROJECT

1.  It reduces the trial-and-error cycle. Instead of building, failing, and trying to figure out what went wrong, students get feedback during the design phase itself.

2.  The validation engine doesn't just say "error." It explains the electrical rule behind the problem, which means students actually learn something from the feedback.

3.  Lecturers spend a lot of lab time catching basic wiring mistakes. Elera automates that, so instructors can focus on higher-level teaching.

4.  There's nothing to install. It runs in a browser, so any student with a laptop and internet access can use it.

5.  Adding new components is straightforward. A developer only needs to write a metadata entry in the library file; the validation and auto-wiring code doesn't need to change.

6.  The project aligns with the Islamic principle of beneficial knowledge (ilm nafi') by making electronics education more accessible to students who may not have easy access to physical hardware.

7.  It supports STEAM education, especially in contexts where schools and universities don't have enough physical Arduino kits for every student.

## SUMMARY

Elera is an intelligent web-based Arduino circuit design assistant built as a Final Year Project at IIUM KICT. The problem is straightforward: existing tools let students build circuits but don't catch mistakes or teach anything when things go wrong. Elera tackles that with four objectives, covering the visual builder, validation engine, auto-wiring, and metadata-driven component library. The scope is focused on Arduino Uno circuits in a client-side web app aimed at beginners. Constraints include the single-board limitation and the fact that validation is rule-based rather than simulation-based. The next chapter reviews the existing tools in detail and identifies exactly where they fall short.

# CHAPTER 2

# LITERATURE REVIEW

## INTRODUCTION

Before building Elera, it was important to understand what tools already exist for Arduino circuit design and where they fall short. A good literature review doesn't just list tools; it helps identify the specific gaps that the proposed system needs to fill. The four platforms reviewed here are Tinkercad Circuits, Fritzing, Wokwi, and Cirkit Designer. Each one takes a different approach to the problem, and each has both strengths and clear limitations.

## SYSTEM REVIEW

Virtual circuit design tools tend to serve very different purposes. Some are built for absolute beginners, others for professional engineers, and most sit somewhere in between. The platforms reviewed below represent the most commonly used options in electronics education today.

### Existing Systems

Tinkercad Circuits

Tinkercad Circuits is a browser-based circuit editor built by Autodesk. It's aimed squarely at beginners, covering primary, secondary, and introductory university-level programmes (Autodesk, 2017). The platform started as a 3D modelling tool in 2011, got acquired by Autodesk in 2013, and then in 2017 Autodesk merged in the "Electronics Lab" (formerly 123D Circuits) to create what's now known as Tinkercad Circuits (Wikipedia, 2024).

[FIGURE 2.1: Tinkercad Circuits Interface, showing the breadboard editor, component toolbox, and code panel.]

It runs entirely in the browser with a drag-and-drop interface. There's a large selection of simulated components, including the Arduino Uno, Micro:bit, and various sensors (Tinkercad, n.d.). It's available in 16 languages and saw a big jump in usage during the COVID-19 pandemic when remote learning became the norm (Tinkercad, n.d.). One of its strongest features is the dual programming interface: beginners can use visual block-based coding while more experienced users can write C++ directly (IoTDunia, 2024). There's also an integration with Autodesk Fusion 360 for users who want to take their designs into professional PCB workflows (Autodesk, 2021).

Research has shown that Tinkercad's visual environment helps with student motivation. Case studies with secondary school students found that the platform made abstract concepts like "smart air conditioning systems" feel engaging and accessible (ERIC, 2024).

Fritzing

Fritzing started in 2007 at the University of Applied Sciences Potsdam in Germany, under Prof. Reto Wettach (Fritzing, n.d.). It was originally meant to help non-engineers, designers, and artists document their electronic prototypes. The Friends-of-Fritzing non-profit organisation maintains it as open-source software (Friends-of-Fritzing, 2009).

[FIGURE 2.2: Fritzing Interface, showing the photorealistic breadboard view.]

What makes Fritzing stand out is its three-view workflow: Breadboard View (photorealistic), Schematics View (abstract circuit diagram), and PCB View (for generating Gerber manufacturing files) (Fritzing, n.d.). The breadboard view uses high-fidelity graphics that look like real components on a real desk, which makes it very intuitive for beginners. The software uses XML and SVG standards, so users can create custom parts through the Parts Editor (Fritzing, n.d.). It's widely used in academic settings like the MIT Media Lab and across maker communities.

Wokwi

Wokwi was introduced around 2019 by Wokwi B.V. and takes a very different approach from Tinkercad or Fritzing (Wokwi, n.d.). It's built for advanced simulation. The engine underneath, AVR8js, is an open-source Arduino simulator written in JavaScript and TypeScript that compiles and runs code directly in the browser (Wokwi Blog, n.d.).

[FIGURE 2.3: Wokwi Interface, showing a simulated circuit with the code editor.]

The board support goes well beyond Arduino. Wokwi simulates ESP32, STM32, and Raspberry Pi Pico alongside the traditional Arduino family (Wokwi, n.d.). A standout feature is real-time internet connectivity: simulated ESP32 devices can connect to virtual WiFi and run MQTT, HTTP, and NTP protocols entirely in the cloud (IoTDunia, 2024). It also provides professional-grade tools like a virtual logic analyser and GDB debugging (Wokwi, n.d.). The platform integrates with VS Code and CI/CD pipelines such as GitHub Actions, and it's been adopted by universities including Harvard, NUS, and Tufts (Wokwi, n.d.).

A study at Jambi University tested Wokwi in basic physics and electronics courses with fifty students. The results showed that 44% of students achieved "Good" science process skills, and there was a significant positive correlation between using the simulator and skill improvement (Cahaya-IC, 2024).

Cirkit Designer

Cirkit Designer is a newer platform founded by Austin Small, a former Google engineer with an electrical engineering background from the University of Pennsylvania (Cirkit Designer, n.d.). He created it out of personal frustration: as an eighth-grader, he tried to build an Arduino RFID pet feeder and spent years on it because no tool could guide him through both the wiring and the code (Cirkit Designer, 2024).

[FIGURE 2.4: Cirkit Designer Interface, showing the AI assistant panel alongside the circuit editor.]

The platform combines a library of over 30,000 components with an integrated Arduino IDE powered by VS Code (Cirkit Studio, n.d.). Its main selling point is an AI assistant that uses Large Language Models (LLMs) to walk users through the whole process. A user can type something like "build an RFID pet feeder," and the AI recommends specific components, explains why each one is needed, drafts Arduino code, includes the right libraries, and helps debug compilation errors (Cirkit Designer, 2024). It supports Arduino, ESP32, and Raspberry Pi boards.

### Advantages

Tinkercad is the most approachable option for absolute beginners. The block-based coding and drag-and-drop interface make it a natural fit for younger learners and introductory courses. The Fusion 360 integration is a nice bonus for anyone who eventually wants to do PCB work.

Fritzing is unmatched for documentation. If the goal is to create clear, visual circuit diagrams that non-technical people can understand, it's the best choice. The fact that it can go from breadboard sketch to PCB manufacturing files in one project is genuinely useful.

Wokwi is by far the most powerful simulator. Support for modern microcontrollers, virtual WiFi, debugging tools, and CI/CD integration put it in a different league. For university-level embedded systems work, it's hard to beat.

Cirkit Designer brings something new with its AI assistant. Being able to describe a project in plain English and get component suggestions and boilerplate code is a real time-saver, and it points to where the industry is heading.

### Disadvantages

For all their strengths, these four platforms share some important weaknesses.

Tinkercad has UI problems that actually get in the way of learning. There's no quick reset function, so clearing the canvas means deleting components one at a time. Worse, when the code blocks panel is open, it physically covers the circuit view, which makes it hard to see what the code is doing to the hardware in real time (PMC, 2023). There's no proactive error checking during design.

Fritzing can't simulate anything at all. It doesn't run code, doesn't show voltage or current, and has no validation of any kind. Users have to build and test their circuits elsewhere and only come back to Fritzing for documentation (KSPU, n.d.). There's also a well-known criticism from the engineering community: complex Fritzing diagrams turn into a "rat's nest" of overlapping wires that are harder to read than proper schematics (Arduino Forum, 2023).

Wokwi is code-centric. It assumes users already know how to program in C++ or MicroPython, and there's no visual block-based option to ease beginners in (IoTDunia, 2024). Several advanced features also sit behind a $25/month paywall, which limits access for students.

Cirkit Designer's AI is impressive, but it has a fundamental reliability issue. Because it runs on probabilistic language models, it can hallucinate: recommending the wrong component, confusing an RFID reader with a tag, or generating wiring instructions that don't account for physical constraints (Cirkit Designer, 2024). The AI also only provides text-based wiring instructions. It tells the user where to place wires but doesn't actually draw them, so the manual effort stays the same.

The biggest gap across all four: none of them check for electrical errors during the design process. None of them tell the user "you're missing a ground connection" or "this LED will burn out without a resistor." And none of them explain why a mistake is a mistake.

## DISCUSSION

[TABLE 2-1: Comparison of Existing Circuit Design Platforms]

| Feature | Tinkercad | Fritzing | Wokwi | Cirkit Designer |
| --- | --- | --- | --- | --- |
| Primary Focus | Beginner education, simulation | Documentation, PCB design | Advanced IoT simulation | AI-assisted prototyping |
| Platform | Web-based | Desktop | Web-based | Web-based |
| Board Support | Arduino Uno, Micro:bit, ATtiny | Visual only (no simulation) | Arduino, ESP32, STM32, Pi Pico | Arduino, ESP32, Raspberry Pi |
| Coding Interface | Block-based + C++ | None | C++, MicroPython | C++ (VS Code engine) |
| Real-Time Simulation | Basic | No | Advanced (WiFi, MQTT) | Yes |
| AI Assistance | No | No | No | LLM-based |
| Real-Time Validation | No | No | No | No (AI text guidance only) |
| Auto-Wiring | No | No | No | No (text instructions only) |
| Pedagogical Feedback | No | No | No | Partial (AI-generated) |

Looking at the table, a clear pattern emerges. Each tool is good at something specific: Tinkercad at beginner accessibility, Fritzing at documentation, Wokwi at simulation power, Cirkit Designer at AI-assisted code generation. But they all leave the same holes.

Real-time validation is missing everywhere. All four tools either show errors only after you run a simulation, or they don't validate at all. Nobody checks the wiring while the student is still building.

Auto-wiring doesn't exist. Every tool requires manual wire routing. Cirkit Designer's AI gives text instructions ("connect SDA to pin A4"), but the user still has to click and drag each wire themselves.

Educational feedback is an afterthought. When things go wrong, the feedback is either a cryptic compiler error, a generic simulation failure, or nothing at all. None of these tools try to teach the student what the underlying electrical problem is.

Components are just graphics. In most of these tools, a virtual LED is just a picture with connection points. It doesn't carry metadata about its current draw, pin roles, or voltage requirements. Without that kind of structured data, automatic validation and wiring aren't really possible.

Elera was designed to fill these four gaps. It combines a visual circuit builder with a deterministic validation engine (nine predefined electrical rules), an auto-wiring engine that assigns pins based on type, and a metadata-driven component library that makes the whole system extensible. Unlike Cirkit Designer's probabilistic AI approach, Elera uses fixed rules, so the validation is predictable and can't hallucinate. And unlike all four tools reviewed, Elera gives context-aware feedback that explains both the problem and how to fix it.

# CHAPTER 3

# METHODOLOGY

## INTRODUCTION

Getting a clear picture of how the system was planned and designed matters just as much as the final product. The choices made during the methodology phase, from the development model to the data structures, directly shaped how Elera turned out. This chapter walks through the development approach, the requirements that were gathered, the logical design expressed in standard diagrams, the data model behind the application, and the user interface prototype.

## DEVELOPMENT APPROACH

Elera was built using an Agile Iterative approach. In practice, that meant breaking the project into short development cycles (sprints), where each one focused on a specific feature and produced a working piece of the system. After each sprint, the output was tested, and any issues found were fed into the next cycle.

There were a few reasons for choosing this over a more traditional approach like Waterfall:

•  The requirements weren't fully clear from the start. As a single-developer FYP, the exact validation rules, pin assignment logic, and metadata schema became clearer as the developer worked through each feature. An iterative model handles that kind of gradual refinement well.

•  Each sprint delivered something functional. Even halfway through the project, there was always a working version to demonstrate, which reduces the risk of showing up at the end with a broken prototype.

•  It keeps things manageable. Instead of planning everything upfront and hoping it works, the developer could focus on one module at a time.

The project was organised into seven sprints:

1.  Interactive canvas with component placement, grid snapping, and pan/zoom.

2.  Component library with metadata and the sidebar interface.

3.  Manual pin-to-pin wiring with orthogonal routing.

4.  Validation engine (nine rules) and the validation bar UI.

5.  Auto-wire engine with smart pin assignment.

6.  State management upgrades: undo/redo, localStorage persistence, anti-overlap collision detection.

7.  Integration testing, UI polish, and wire cleanup features.

[FIGURE 3.1: Agile Iterative Development Model, showing the sprint cycle: Plan, Design, Build, Test, Review, repeating for each sprint.]

## REQUIREMENTS SPECIFICATION

Requirements were gathered through three methods. First, a literature review of existing tools (covered in Chapter 2) helped identify what features are standard and what's missing. Second, informal observation of students making common wiring mistakes during lab sessions gave practical insight into what errors the system should catch. Third, early prototyping of the UI and validation logic revealed which rules worked well and which needed adjustment.

### Functional Requirements

FR1: Component Placement and Management

•  FR1.1: Users can drag components from the sidebar and drop them onto the canvas.

•  FR1.2: Placed components snap to a 20-pixel grid so things stay aligned.

•  FR1.3: With anti-overlap mode on, the system nudges components that would stack on top of each other.

•  FR1.4: Users can select, reposition, and delete components.

FR2: Component Library and Sidebar

•  FR2.1: Components are shown in a categorised sidebar with six groups: Boards, Sensors, Input, Output, Actuators, and Passive.

•  FR2.2: A search box at the top filters the list as the user types.

•  FR2.3: Each entry shows an icon, name, and short description.

FR3: Manual Wiring

•  FR3.1: Wires are created by clicking a source pin, then clicking a destination pin.

•  FR3.2: Wire paths follow orthogonal (right-angle) routing.

•  FR3.3: Colour coding is automatic based on signal type (red for power, black for ground, and so on).

•  FR3.4: Users can add, drag, and delete wire waypoints for manual route adjustments.

•  FR3.5: Individual wires can be selected and deleted.

FR4: Real-Time Validation

•  FR4.1: Validation runs automatically every time a structural change happens (adding, removing, or wiring a component).

•  FR4.2: The engine checks for nine categories of errors:

•  (a) No Arduino board on the canvas.

•  (b) An LED wired without a current-limiting resistor.

•  (c) Total current draw going over the Arduino's 500 mA USB limit.

•  (d) Two or more signal components sharing the same Arduino I/O pin.

•  (e) A servo motor connected to serial pins 0 or 1.

•  (f) A component that's missing its power (VCC) or ground (GND) connection.

•  (g) An I2C device not wired to the correct pins (A4 for SDA, A5 for SCL).

•  (h) A component that's been placed but has no wires at all.

•  (i) A partially wired component with pins left floating.

•  FR4.3: Each result includes a severity level (error, warning, or info), a message explaining the issue and the fix, and an icon.

•  FR4.4: Results appear in a validation bar panel overlaid on the canvas.

FR5: Intelligent Auto-Wiring

•  FR5.1: Users can trigger auto-wire to connect a component to the Arduino automatically.

•  FR5.2: Pin assignment follows the metadata rules: PWM pins for servos, analogue pins for potentiometers, I2C pins for LCDs, and digital pins for most other components.

•  FR5.3: The engine tracks which Arduino pins are already taken and avoids conflicts.

•  FR5.4: When multiple pins of the same type are free, the engine picks the one that's physically closest to keep wires short.

FR6: State Management

•  FR6.1: Undo and redo work across up to 50 history states.

•  FR6.2: Projects auto-save to localStorage 500 milliseconds after each structural change.

•  FR6.3: On page reload, the last saved project is restored automatically.

•  FR6.4: A "Clear All" button wipes the canvas and resets everything.

[TABLE 3-1: Summary of Functional Requirements, listing IDs FR1.1 through FR6.4 with descriptions and corresponding modules.]

### Non-Functional Requirements

NFR1 (Performance): Validation results should show up within 100 milliseconds of any change. The feedback needs to feel instant.

NFR2 (Usability): A student with no prior simulation tool experience should be able to pick it up. Error messages should be written in plain language, not technical jargon.

NFR3 (Extensibility): Adding a new component should only mean writing one metadata entry in the library file. The validation and auto-wire code shouldn't need any changes.

NFR4 (Compatibility): The system should work on current versions of Chrome, Firefox, and Edge.

NFR5 (Reliability): User work shouldn't be lost. Auto-save via localStorage means the project survives page refreshes and accidental tab closures.

## LOGICAL DESIGN

### System Analysis and Design Diagrams

System Architecture

Elera is built on a modular architecture using Lit 3 Web Components. There are ten modules, and they communicate through an event-driven pattern. When a user does something on the canvas (say, placing a wire), the Store updates its internal state and fires a "structural-change" event. The Validation Engine listens for that event, re-checks the circuit, and sends the results to the Validation Bar, which re-renders. The user sees updated feedback without any manual refresh.

Here's what each module does:

| Module | Role |
| --- | --- |
| Circuit App | Root layout component. Sets up the header toolbar, sidebar, canvas, and validation bar. |
| Component Library | The metadata catalog. Stores pin types, current ratings, auto-wire rules, and code templates for every supported component. |
| Component Sidebar | Renders the categorised, searchable component list. Handles drag events. |
| Circuit Canvas | SVG-based interactive surface. Handles placement, wiring, selection, pan/zoom, and waypoint editing. |
| Placed Component | Renders a single component instance on the canvas. Exposes pin targets and reports pin positions to the Store. |
| Circuit Store | Central state manager. Holds all instances, wires, selections, viewport state, undo/redo history, and handles localStorage persistence. |
| Validation Engine | Runs nine electrical rules against the current circuit state. Returns arrays of errors, warnings, and info messages. |
| Validation Bar | Displays validation results with colour-coded severity (red, amber, blue) and icons. |
| Auto-Wire Engine | Assigns Arduino pins to component pins using type-based heuristics. Creates wire entries in the Store. |
| Wire Path Utility | Calculates orthogonal wire routing paths and generates obstacle-aware waypoints. |

[FIGURE 3.2: System Architecture Diagram. A block diagram showing the ten modules and data flow between them. Component Library feeds metadata to Sidebar, Canvas, Validation Engine, and Auto-Wire Engine. Canvas and Auto-Wire Engine read/write to Store. Store fires structural-change events to Validation Engine. Validation Engine outputs to Validation Bar.]

Use Case Diagram

The primary actor is the Student. The supported use cases are: Place Component, Move Component, Delete Component, Wire Components, Edit Wire Waypoints, Search Components, Auto-Wire Component, View Validation Results, Undo/Redo, and Clear Project.

[FIGURE 3.3: Use Case Diagram. UML use case diagram with the Student actor connected to ten use cases.]

Activity Diagram

[FIGURE 3.4: Activity Diagram for the circuit building workflow. Start → Place Arduino Uno → Place a Component → Choose Manual Wire or Auto-Wire → Manual path: click source pin, click destination pin, wire appears → Auto-Wire path: trigger engine, pins assigned, wires generated → Validation runs automatically → View results → Errors present? Yes: fix and loop back. No: circuit complete → End.]

Sequence Diagram

[FIGURE 3.5: Sequence Diagram for the validation flow. User interacts with Canvas (places a wire) → Canvas calls Store.completeWiring() → Store adds wire, fires 'structural-change' event → Validation Engine receives event, calls validateCircuit(), builds context, runs nine rules → Returns results to Validation Bar → Bar renders messages → User reads feedback.]

Package Diagram

[FIGURE 3.6: Package Diagram showing module organisation. Three packages: UI Components (circuit-app, circuit-canvas, placed-component, component-sidebar, validation-bar), Services (validation-engine, auto-wire-engine), and Data (store, component-library, wire-path). UI Components depends on Data. Services depends on Data.]

### Data Flow Diagram

Context Diagram (DFD Level 0)

One external entity (the Student) interacts with the Elera system. Inbound data flows include component placement commands, wiring actions, search queries, and auto-wire requests. Outbound flows include the visual circuit display, validation results, and auto-wire confirmations.

[FIGURE 3.7: Context Diagram showing Student entity, Elera process, and labelled data flows.]

DFD Level 1

[FIGURE 3.8: Level 1 DFD with four processes. (1) Manage Components: takes placement/deletion commands, reads/writes to the Circuit Store data store. (2) Manage Wires: takes wiring commands, reads/writes wire data. (3) Validate Circuit: triggered by structural changes, reads instance and wire data from Store, reads metadata from Component Library, outputs validation results. (4) Auto-Wire: takes auto-wire requests, reads metadata and used-pin data, writes new wires to Store.]

## DATABASE DESIGN

Elera doesn't use a traditional database in this phase. It's a fully client-side application, so all state lives in memory within the Circuit Store. The Store is a JavaScript class that extends EventTarget, which gives it a built-in event system for notifying UI components about changes.

The main data structures are:

Instances Array (one entry per placed component):

| Attribute | Type | Description |
| --- | --- | --- |
| id | String | Unique identifier, e.g. "inst_1" |
| componentId | String | Links to the library definition, e.g. "led" or "servo" |
| x | Number | Canvas X position (grid-snapped) |
| y | Number | Canvas Y position (grid-snapped) |

Wires Array (one entry per wire connection):

| Attribute | Type | Description |
| --- | --- | --- |
| id | String | Unique identifier, e.g. "wire_1" |
| from.instanceId | String | Instance ID of the source component |
| from.pinName | String | Pin name on the source component |
| to.instanceId | String | Instance ID of the destination component |
| to.pinName | String | Pin name on the destination component |
| waypoints | Array | Intermediate {x, y} routing points |
| color | String | Auto-assigned based on signal type |

Pin Info Map: a Map keyed by instance ID, holding arrays of pin position and signal data that each rendered component reports back.

Other state: the current wiring-in-progress (if any), which component or wire is selected, viewport zoom and pan offsets, and the undo/redo history stack (capped at 50 snapshots).

For persistence, the Store serialises the instances and wires arrays to JSON and saves them in localStorage. Saving is debounced to 500 ms after each structural change, so it doesn't fire on every tiny update.

[TABLE 3-2: Circuit Store Data Model summary.]

Planned database for FYP2:

In the next phase (INFO 4402), a PostgreSQL database is planned with tables for Users, Projects, ComponentInstances, and Wires. That will allow cloud-based storage, multi-device access, and project sharing.

## USER INTERFACE / PROTOTYPE DESIGN

Elera's interface uses a three-panel layout. There's a fixed header bar at the top, a sidebar on the left for browsing components, and a large central canvas where the actual circuit design happens. The validation bar sits as an overlay inside the canvas area.

### Header Toolbar

The header shows the Elera branding and subtitle ("Intelligent Arduino Circuit Builder"). On the right side, there are toolbar buttons for undo, redo, wire cleanup, wire reset, anti-overlap toggle, and a clear button. It's compact and stays out of the way.

[FIGURE 3.9: Header Toolbar screenshot.]

### Component Sidebar

A fixed-width panel (240 pixels) on the left. At the top is a search box that filters components in real time as the user types. Below that, components are grouped into six categories: Boards, Sensors, Input, Output, Actuators, and Passive. Each entry shows an emoji icon, the component name, and a one-line description. To place a component, the user just drags it from the sidebar and drops it onto the canvas.

[FIGURE 3.10: Component Sidebar screenshot.]

### Circuit Canvas

The canvas is the main workspace. It's an SVG element that supports drag-and-drop placement, click-to-select, pin-to-pin wiring, pan (middle-click drag or trackpad), zoom (scroll wheel), grid-snapped movement, and wire waypoint editing.

Components are rendered using @wokwi/elements, which provides realistic SVG visuals of actual Arduino parts. Each component shows interactive pin targets (small circles that highlight on hover) to indicate where wires can connect.

[FIGURE 3.11: Circuit Canvas screenshot with an Arduino Uno and several components wired up.]

### Validation Bar

An overlay panel inside the canvas area. It shows a scrollable list of validation messages, each with an icon and colour-coded background: red for errors, amber for warnings, blue for informational notes. It updates automatically whenever the circuit changes, so the user always sees the current state.

[FIGURE 3.12: Validation Bar screenshot with example messages like "LED has no current-limiting resistor" and "HC-SR04 is missing a ground connection."]

### Auto-Wire

When a user triggers auto-wire on a component, the engine creates wires from the component's pins to the right Arduino pins. The wires show up immediately on the canvas, and the validation bar refreshes to reflect the updated circuit.

[FIGURE 3.13: Auto-Wire Result screenshot, showing a servo motor with auto-generated wires going to PWM, VCC, and GND on the Arduino.]

### Navigation Flow

[FIGURE 3.14: Navigation Flow diagram. App loads → Canvas (empty or restored from localStorage) → User can: place a component, search, wire pins, auto-wire, view validation, undo/redo, or clear. All paths lead back to the canvas.]

As required by the FYP guidelines, the interface screens shown above represent the implemented prototype at full completion for FYP1.

## SUMMARY

The development of Elera followed an Agile Iterative model across seven sprints, starting from the core canvas and building up to the validation and auto-wire engines. Requirements were gathered from a literature review, observation, and prototyping, and they cover six modules with over 20 functional requirements and five non-functional ones. The system architecture is modular, with ten components communicating through a centralised store and event-driven updates. All state is managed client-side in memory with localStorage persistence. The UI prototype is fully functional and matches the three-panel layout described here. In FYP2, the focus will shift to implementation evaluation, user testing, and adding server-side persistence.

# REFERENCES

Autodesk. (2017). *Tinkercad Circuits*. https://www.tinkercad.com

Autodesk. (2021). Autodesk Fusion 360 Electronics Sees Exciting Improvements. https://www.autodesk.com/products/fusion-360/blog/fusion-360-electronics-new-features-november-2021/

Arduino Forum. (2023). BEGINNERS:: Why are Fritzy diagrams so unpopular? https://forum.arduino.cc/t/beginners-why-are-fritzy-diagrams-so-unpopular/1161236

Cahaya-IC. (2024). Utilization of Wokwi Technology as a Modern Electronics Learning Media. *Journal of Education Technology and Literacy Culture*. https://cahaya-ic.com/index.php/JETLC/article/view/1392

Cirkit Designer. (2024). Introducing Cirkit Designer: an AI-powered IDE for building Arduino and circuit projects. *Medium*. https://medium.com/@cirkit-designer/introducing-cirkit-designer-an-ai-powered-ide-for-building-arduino-and-circuit-projects-3fafb519b9bd

Cirkit Designer. (n.d.). *Welcome to Cirkit Designer*. https://learn.cirkitdesigner.com/

Cirkit Studio. (n.d.). *Cirkit Designer, Design, Simulate and Prototype Circuits*. https://www.cirkitstudio.com/

ERIC. (2024). Effect of Tinkercad on Students' Computational Thinking Skills and Perceptions. https://files.eric.ed.gov/fulltext/EJ1290797.pdf

Friends-of-Fritzing. (2009). *Fritzing*. https://fritzing.org

Fritzing. (n.d.). Open-source software for documenting prototypes. https://fritzing.org/media/uploads/publications/FritzingInfoBrochure-lowRes.pdf

IoTDunia. (2024). Top 5 Best IoT Simulation Tools Online (No Hardware Needed). https://iotdunia.com/best-iot-simulation-tools-online/rbsnp/

KSPU. (n.d.). Specialized Applied Software Fritzing and its Use for Education. *Information Technologies in Education*. https://www.ite.kspu.edu/index.php/ite/article/view/767

PMC. (2023). Examination of the usability of Tinkercad application in educational environments. *PLOS ONE*. https://pmc.ncbi.nlm.nih.gov/articles/PMC10034876/

ResearchGate. (2022). Design and Technology in Malaysian Secondary Schools: A Perspective on Challenges. https://www.researchgate.net/publication/357716280

Tinkercad. (n.d.). *Official Guide to Tinkercad Circuits*. https://www.tinkercad.com/blog/official-guide-to-tinkercad-circuits

Wikipedia. (2024). Tinkercad. https://en.wikipedia.org/wiki/Tinkercad

Wokwi. (n.d.). *Wokwi, World's most advanced ESP32 Simulator*. https://wokwi.com/

Wokwi Blog. (n.d.). Brilliant Things People Built with AVR8js Arduino Simulation. https://blog.wokwi.com/cool-things-people-built-with-avr8js/

Xu, J., et al. (2026). CircuitLM: A Multi-Agent LLM-Aided Design Framework for Generating Circuit Schematics from Natural Language Prompts. *arXiv*. https://arxiv.org/abs/2601.04505

# APPENDICES

(Gantt chart and additional materials to be added)